Rudders

It is an underwater appendage the surface of which is used to generate sufficient force to steer the ship when it is in motion.  A single rudder is fitted in ships with single tail shaft or single propeller.  Twin rudders, i.e. two rudders, are fitted on ships with twin screws or twin propeller arrangement.

# Construction of a typical rudder for merchant ships:

- Normally made from standard streamlined profiles.

- A typical rudder will be constructed of plates stiffened by horizontal and vertical webs.

- A coupling palm is normally provided at the top of the rudder for attachment to the rudder stock.

- Lifting eyes are provided for lifting and dismantling of the rudder.

- A drain plug at the top and another at the bottom are provided for drainage, coating and testing purposes.

- A closing plate is fitted on one side of the rudder. This closing plate is the last piece of plate to be fitted after carrying out internal weldings of the rudder. Since only the cut edges of the closing plate can be welded from the outside, additional slots or plugs must be cut on the closing plate so that welding to the horizontal and vertical webs can be effected.